Swirl Stabilized Flames

Summary and recommendations for

TNF7

Masri, A.R., Al-Abdeli, Y.M. and Kalt, P.A.K.

3 November 2004

There is a comprehensive data set that is now made available on the web for a range of swirling flow conditions, both reacting and non-reacting. These data include detailed boundary conditions including time series of velocity, flow, mixing and composition fields presented in the form of spontaneous data as well as radial and conditional means.

Swirling flows are complicated due to the existence of vortex breakdown and recirculation. Flames may have single or double recirculation zones depending on a range of factors and the table below shows conditions where these occurs. Also, swirling flows are inherently unstable and non-reacting jets enter into a mode of precession when the swirl number exceeds a certain threshold. Swirling flows may have more than one mode of instability and, depending on the flow condition, may experience events local extinction and re-ignition. The precession is challenging to compute and the finite rate chemical kinetic effects add a level of complexity to the chemistry requirement. Asymmetry is another characteristic that exists under certain flow conditions.

Tables 1 and 2 below give a summary of the characteristics of the flames as obtained from a series of experiments conducted as Sydney University and Sandia. More details about these flows and flames may be found on the web, in papers and in the PhD thesis by Y.M. Al-Abdeli.

| Flame | Fuel Mixture (vol. ratio) | Us (m/s) | Ws (m/s) | Uj (m/s) | Re jet (-) | Sg (-) | UBO (m/s) | Lf (m) | Uj/UBO (%) | Local Extinction | Asymmetry | 2nd RZ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SM1 | CNG | 38.2 | 19.1 | 32.7 | 7200 | 0.5 | 166 | 0.12 | 20 | Little |  | 65-110 |
| SM2 |  | 38.2 | 19.1 | 88.4 | 19500 | 0.5 | 166 | 0.18 | 53 | Lots |  | 65-145 |
| SMA1 | CNG- air (1:2) | 32.9 | 21.6 | 66.3 | 15400 | 0.66 | 241 | 0.2 | 28 | Lots | Yes |  |
| SMA2 |  | 16.3 | 25.9 | 66.3 | 15400 | 1.59 | 216 | 0.23 | 31 | Little |  |  |
| SMA3 |  | 16.3 | 25.9 | 132.6 | 30800 | 1.59 | 216 | 0.3 | 61 | Some | Yes |  |
| SMH1 | CNG- H2 (1:1) | 42.8 | 13.8 | 140.8 | 19300 | 0.32 | 267 | 0.37 | 53 | Little |  | 110-160 |
| SMH2 |  | 29.7 | 16 | 140.8 | 19300 | 0.54 | 281 | 0.4 | 50 | Little |  |  |
| SMH3 |  | 29.7 | 16 | 226 | 31000 | 0.54 | 281 | 0.5 | 80 | Lots | Yes |  |

Table 1: Selected properties of the swirl stabilized flames.

The purpose of this note is to make a recommendation as to the best path to approach the computation of these flames in increasing order of complexity. Individual groups should make their own decisions depending on their priorities and the capabilities of their codes. The order followed here is to start with simple, symmetric, non-precessing flames flames that are relatively far from blow off. Then proceed to flames closer to extinction and then precessing flames. Table 3 gives the recommended order.

Table 2: Precession frequencies as obtained from three separate experiments

| Flame Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Flame | Centre jet | Sg | LDV | Mie | Shadowgraph |
| SM1 | CNG | 0.5 | more tests are warranted |  |  |
| SM2 |  | 0.5 |  |  |  |
| SMA1 | CNG-air (1:2) | 0.66 |  |  |  |
| SMA2 |  | 1.59 | 7-29 Hz | - | 28 Hz (Mode I) |
| SMA3 |  | 1.59 | 57-67 Hz | 49-53 Hz (Mode I) | 62 Hz (Mode I) |
| SMH1 | CNG-H2 (1:1) | 0.32 | 41-58 Hz | 47 Hz (Mode I) | 47 Hz (Mode I) |
| SMH2 |  | 0.54 | 48-58 Hz | 55 Hz (Mode I) | 54 Hz (Mode I) |
| SMH3 |  | 0.54 | 63-67 Hz | - | 60 Hz (Mode II) |

Table 3: Recommendation for order of calculations and key characteristics

| Order | Flame | Re | S | 2nd RZ | Local Extinction | Pre- Cession | Asym metry |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | SM1 | 7200 | 0.5 | yes | little | no | no |
| 2 | SMA2 | 15400 | 1.59 | no | little | no | no |
| 3 | SM2 | 19500 | 0.5 | yes | lots | no | no |
| 4 | SMA1 | 15400 | 0.66 | no | lots | no | yes |
| 5 | SMA3 | 30800 | 1.59 | no | some | yes | yes |
| 6 | SMH1 | 19300 | 0.32 | yes | little | yes | no |
| 7 | SMH2 | 19300 | 0.54 | no | little | yes | no |
| 8 | SMH3 | 31000 | 0.54 | no | lots | yes | yes |

References:

- www.aeromech.usyd.edu.au/thermofluids

- Kalt, P.A.M., Al-Abdeli, Y.M., Masri, A.R., and Barlow, R.S., 'Swirling Turbulent Non-premixed Flames of Methane: Flowfield and Compositional Structure', Proc. Combust. Inst. 29:1913-1919 (2002).

- Al-Abdeli, Y.M., and Masri, A.R., 'Recirculation and Flow Field Regimes of Unconfined Non-Reacting Swirling Flows', Experimental Thermal and Fluid Sciences, 27(5):655-665 (2003).

- Al-Abdeli, Y.M., and Masri, A.R., 'Stability Characteristics and Flow Fields of Turbulent Swirling Jet Flows', Combust. Theory and Modeling, 7:731-766  (2003).

- Masri, A.R., Kalt, P.A.M., and Barlow, R.S., 'The Compositional Structure of Swirl-Stabilised Turbulent Non-premixed Flames’, Combust. Flame, 137:1-37 (2004).

- Al-Abdeli, Y.M., and Masri, A.R., ‘Precession and Recirculation in Turbulent Swirling Isothermal Jets’, Combust. Sci. Technol. 176:645-665 (2004).